The Target volume is empty (the blue dropdown is blank) — that means Thonny cannot detect your Pico W in bootloader mode. Follow these steps:

Step 1 — Put Pico W into bootloader mode correctly

- Unplug the USB cable from your Pico W completely

- Hold down the BOOTSEL button on the Pico W (small white button)

- While holding BOOTSEL, plug the USB cable back in

- Hold for 2-3 seconds then release BOOTSEL

Step 2 — Check it appears as a drive

Open File Explorer — you should see a new drive appear called RPI-RP2 like a USB flash drive. If you see it, the Pico is in bootloader mode correctly.

Step 3 — Go back to Thonny

Once RPI-RP2 appears in File Explorer, go back to the Install dialog — the Target volume dropdown should now show RPI-RP2. Select it and click Install.

If RPI-RP2 still doesn't appear:

- Try a different USB cable — many cheap cables are charge-only with no data wires

- Try a different USB port on your laptop

- Make sure you are holding BOOTSEL before plugging in, not after

The most common reason this fails is a charge-only USB cable — that is the first thing to check.

## You said: you said something like this i didnt understand :4.

you said something like this i didnt understand :4. Upload files in this order: - firmware/config.py - firmware/sensors/mq2.py - firmware/sensors/mq135.py - firmware/sensors/dht.py - firmware/mqtt_client.py - firmware/alert.py - firmware/display.py - firmware/main.py ← upload this last 5. Press the Run button (or reset the Pico)

6:18 PM

## Claude responded: After flashing MicroPython onto the Pico W, you need to copy your firmware files onto it using Thonny.

After flashing MicroPython onto the Pico W, you need to copy your firmware files onto it using Thonny. Here is exactly what to do step by step:

Step 1 — Open Thonny and connect to Pico W

Make sure Thonny shows "MicroPython (Raspberry Pi Pico)" in the bottom right corner. If it shows your Windows Python, click it and switch to Pico.

Step 2 — You will see two panels in Thonny

Left/Top panel   = your laptop files

Bottom panel     = Pico W files (MicroPython shell)

Go to View → Files to open the file browser panel showing both your laptop and the Pico side by side.

Step 3 — Create the sensors folder on Pico first

In the Pico side of the file panel, right click → New Directory → name it sensors

Step 4 — Upload each file one by one

On your laptop side navigate to your saniguard/firmware/ folder. Then for each file, right click on it → "Upload to /":

Right click config.py       → Upload to /

Right click mqtt_client.py  → Upload to /

Right click alert.py        → Upload to /

Right click display.py      → Upload to /

Then go into your sensors/ folder on the laptop side and upload each sensor file → "Upload to /sensors":

Right click mq2.py    → Upload to /sensors

Right click mq135.py  → Upload to /sensors

Right click dht.py    → Upload to /sensors

Then finally upload the main file last:

Right click main.py   → Upload to /

Step 5 — Verify on Pico side

Your Pico W file structure should look like this:

/ (root)

├── config.py

├── main.py

├── mqtt_client.py

├── alert.py

├── display.py

└── sensors/

├── mq2.py

├── mq135.py

└── dht.py

Step 6 — Run it

Click the green Run button in Thonny or press F5. You should see in the shell at the bottom:

[WiFi] Connecting to YourWiFi ...

[WiFi] Connected. IP: 192.168.x.x

[MQTT] Connected to 10.2.24.158

[Sensor] smoke=... gas=... air=... temp=... hum=...

main.py is uploaded last because Pico W automatically runs main.py on boot — if it ran before the other files were uploaded it would crash with import errors.